# Path Safety Assessment Tool (PSAT) User Guide

Welcome to PSAT. The tool helps you create projects from geotagged survey images, review coded path segments, calculate safety risk scores, analyse safety hazards and test treatments.

## Table of Contents

- 1.1 Open the Project List

  - 1.2 Create a Project

  - 1.3 Navigate Between Workflows

  - 1.4 Using the Help Guide

  - 1.5 Viewing and Updating GIS Layers

## 1. Getting Started

### 1.1 Open the Project List

Use the Home page to browse all projects. You can:

- search by project name or road name

- filter by tags (e.g. NSC, AMK)

- sort by project name, verification progress, distance verified, autocode progress, and last modified time

- select one or more projects for deletion, coding, path analysis, or treatment work

### 1.2 Create a Project

Use Create Project when you want to build a new project from the source image folders in in/.

You can create a project in three ways:

- Single folder — select one source folder directly

- Polygon / Planning Area — draw a polygon or click a planning area on the map and create a project from multiple selected roads

- Upload area GIS layers — upload area layers to define the project boundary

When creating a project:

- enter a project name without underscores

- add tags if needed (for easy grouping, e.g. NSC, AMK)

- upload images into a source folder if the folder is missing

- check that selected roads are marked as available before you create the project

#### Road Highlight on Source Folder Select

When you select a source folder in the Single Folder workflow, PSAT reads the road name from the folder and automatically highlights the matching road on the map in amber/orange. The map also pans and zooms to that road so you can visually confirm you are creating the project from the correct location before proceeding.

### 1.3 Navigate Between Workflows

From the project list, you can send selected projects for the following:

| Workflow | Purpose |
| --- | --- |
| Delete Project | Housekeeping to remove unwanted projects |
| Coding | Detailed attribute review and saving |
| Analyse Projects | Multi-project filtering, charts, generate reports and export data, images, shp files |
| Treatment Application | Before/after scenario testing |

### 1.4 Using the Help Guide

Click the Help (?) button (available from any page on the top left corner) to open the in-app guide. The Help page contains three tabs:

- User Guide — step-by-step instructions for all workflows (this guide)

- Admin Guide — system deployment, model management, and infrastructure

- Developer Guide — technical architecture, API reference, and scoring logic

### 1.5 Viewing and Updating GIS Layers

Click View GIS Layers from the sidebar to open the GIS Layers dashboard.

From the GIS Layers page you can:

- Browse all current GIS layers (sorted in alphabetical order)

- View last updated date, required columns and metadata for each layer

- Using the Update GIS Layer,

  - Add entirely new GIS layers for new categories

  - Replace an existing layer with an updated file

  - Delete a layer that is no longer needed

For full GIS layer management instructions, see the GIS Layer Management Guide section.